|  | Data Type | #Stations/receivers | #Sources/transmitters | Station/line spacing | Footprint | Expected Depth penetration | Location/Acquisition time | Status | Notes | Contact Person |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPB | 3D seismics | 2735 Source Points 4334 Receiver points | 1 vibrator : “EnviroVib“ | Rcv : 75m btw line, 25m btw rcv Src: 75m btw line, 37.5m btw src | 7.5 km^2 (2.5x3km) | ~1km (probably more) | Lagoa Salgada; June 16 -July 7, 2025 | Acquisition completed, processing ongoing | 11 days of passive data as by-product | M. Malinowski (processing), P. Jouve (acquisition) |
|  | DAS | N/A (depends on processing) | Same as 3D seismics on first 5 days | N/A | 6.5 km cable in spiral pattern, into two boreholes (359 and 295 m deep) | ?? | Lagoa Salgada, June 23-27, 2025 (co-located with 3 D seismics) | Acquisition completed | 6 TB of data | F. Bretaudeau, M. Malinowski |
|  | MT (include or not??) | 52 (21 in profile, 31 in 3D seismic area) | N/A | ~2.5-3 km on profile, ?? in zoom-in | 50-60 km long profile; zoom-in much smaller | Target crustal structure | Lagoa Salgada (profile extends further N+S), ?? | Acquisition completed, inversions ongoing | Not strictly UNDERCOVER data (UM student field campaign) | M. Becken |
|  | Semi-airborne EM |  |  |  |  |  |  | Planned for 2026 |  | M. Becken |
|  | Passive seismic (HVSR) | 400 (planned) | N/A | ~400 m | ? | Few 100m | Lagoa Salgada | planned Nov/Dec 2025 | 11 test sites already acquired | J. Borges |
| KaCB | Semi-airborne EM | N/A | 8 loops | 200m line spacing | ~22 km² | Uppermost 1 km | KaCB (Heimat Farm); March 2025 | Acquisition, processing completed, inversion on the way | Plans for drone AFMAG? | M. Becken |
| KuSB | MT | 230 new, ~200 pre-existing | N/A | ?? | ~180x150 km | Crust + uppermost mantle | Kuusamo region, June/July 2025 | First part of acquisition completed, first model exists | Second acquisition trip planned summer 2026 | G. Hill, E. Roots, U. Autio |
|  | Regional passive seismic | 35 | N/A | 25-30 km | ~180x150 km (co-located with MT) | Entire Crust | Kuusamo region, May-Nov 2025 | Acquisition completed, processing yet to start |  | C. Sippl, E. Rodriguez |
|  | Large-N passive seismic | ~500 | N/A | 1-1.2 km | ~25x30 km | Uppermost few km | Kitka region, June-August 2025 | Acquisition complete, processing started |  | C. Sippl, E. Rodriguez |
|  | AFMAG | N/A | N/A | 1 km line spacing | 1600 km^2 | Uppermost 2 km | Kitka region (co-located with large-N passive seismic); Aug/Sep 2025 | Acquisition done, processing runs |  | A Thiede, M. Schiffler |
|  | Semi-airborne EM | N/A | 6 grounded electric dipoles | 250 m line spacing | 16.5 km^2 | 1-2 km | Juomasuo area; Aug/Sep 2025 | Acquisition, processing done, inversion ongoing |  | A Thiede, M. Schiffler |
|  | CSEM profile (ExtrEM) | 30 | 3 | ~2-2.5 km? | ~72 km long profile | ~10 km | ~N-S across Kitka area | Measurements done, processing started |  | F. Bretaudeau, C. Patzer |
|  | 3D CSEM/IP | 100 | ~30 | ? | ~3x3 km | ?? | Juomasuo area (co-located with semi-airborne) | Measurements done, processing started |  | F. Bretaudeau, C. Patzer |